Safe Work Method Statement (SWMS)

| Organisation Details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Organisation Name: |  |  |  | Botany Access Pty Ltd |  | Contact Name:: |  |  |  | Justin Brownbill |  |  |  |  |  |
| ABN |  |  |  | 87 073 330239 |  | Contact Position: |  |  |  | Managing Director |  |  |  |  |  |
| Address: |  |  |  | 1729 Botany Rd Botany NSW 2019 |  | Contract Phone No: |  |  |  | 02 9663745 |  |  |  |  |  |
| Project Details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project: |  |  |  |  |  |  |  |  | Area: All |  |  |  |  |  |  |
| Activity: |  |  |  | General Lifting using Telecsopic Handler |  |  |  |  | This SWMS has been developed in consultation with: Justin Brownbill (Director) Reviewed by: Justin Brownbill Position: Managing Director Date:12/1/17 |  |  |  |  |  |  |
| Resources / Trades Involved: |  |  |  | Tele handler operator with non slewing cranae ticket as a minimum, dogman required for slinging of loads. Engineerind trades |  |  |  |  |  |  |  |  |  |  |  |
| Equipment Used: |  |  |  | Telehandler. |  |  |  |  |  |  |  |  |  |  |  |
| Maintenance checks: |  |  |  | Daily Logbook Checks, Quarterly service intervals and 12 monthly Annual service. |  |  |  |  |  |  |  |  |  |  |  |
| Lifting Equipment Used: |  |  |  | All certified lifting equipment with current 3 monthly inspection tag fitted. |  |  |  |  |  |  |  |  |  |  |  |
| Occupational Health Safety or Environmental Legislation: |  |  |  | NSW OHS Act 2000, NSW OHS Regulation 2001, Environmental Protection ACT 1986 | Codes or Standards applicable to the works: |  | AS 2550, AS 1418. AS 4497, AS 1353, AS 2741, AS 2210, AS 1270, AS 1337, AS 1716, AS 1891 NSW Codes of Practice No. 320, 309, 962, 963, 1310 |  |  |  |  |  |  |  |  |
| Level |  | Description of Consequence or Impact |  |  |  |  |  | Consequence |  |  | Likelihood / Probability |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | L Likely | M Moderate | U Unlikely |  |  |
| H (1) (High level of harm) |  | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. |  |  |  |  |  | H (1) (High) |  |  | 1 | 1 | 2 |  |  |
| M (2) (Medium level of harm) |  | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. |  |  |  |  |  | M (2) (Medium) |  |  | 1 | 2 | 3 |  |  |
| L (3) (Low level of harm) |  | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. |  |  |  |  |  | L (3) (Low) |  |  | 2 | 3 | 3 |  |  |
| Level |  | Likelihood / Probability |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Likely |  | Could happen frequently |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Moderate |  | Could happen occasionally |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unlikely |  | May occur only in exceptional circumstances |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Class/Ranking |  | Description / Requirements |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | Will require detailed pre-planning. Actions will be recorded on a Safe Work Method Statement |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  | Will require operational planning. Actions will be recorded on a Safe Work Method Statement |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | Will require localised control measures |  |  |  |  |  |  |  |  |  |  |  |  |

| Item | Exposure to natural elements, Manual handling, Safety Information & Changes to SWMS / JSRA | Risk | Risk Class/ Ranking | Controls | Name of persons responsible for work |
| --- | --- | --- | --- | --- | --- |
| 1 | Exposure to natural elements | Sunburn & dehydration due to exposure to sun. Sickness & loss of production due to cold weather | ###### 3 | - UV sunscreen to be onsite and used when required. - Fresh drinking water must be available to all personnel - Wear wide brim on hardhat and long clothing to reduce exposure to sun when required. - Ensure adequate clothing worn to suit cold conditions. - Ensure operator warms up prior to manual activity. | Tele Handler Operator Botany Access Supervision |
| - 2 | - Manual handling | Injury to personnel | ###### 2 | - Follow manual handling procedures - Team lift where appropriate - Only carry out lifts if comfortable to do continue | Tele Handler operator/ rigger/ trades. |
| - 3 | - Safety Information | Failure to have adequate emergency equipment, PPE or amenities. Failure to know evacuation points. | ###### 2 | - Ensure first aider onsite at all times - PPE required to be worn at all times – Hard hat, Safety boots, Hi Visibility vest or clothing are mandatory & other PPE where required. - Ensure all evacuation points, site amenities and first aid points are known prior to entering site | Tele Handler Operator Botany Access Supervision |

| Item | Exposure to natural elements, Manual handling, Safety Information & Changes to SWMS / JSRA | Risk | Risk Class/ Ranking | Controls | Name of persons responsible for work |
| --- | --- | --- | --- | --- | --- |

| - 4 | - Changes to SWMS / JSRA | Lack of communication of changes made to SWMS / JSRA | ###### 2 | - Consult with workforce on changes or amendments to be made to SWMS / JSRA. Toolbox SWMS / JSRA with workforce and ensure everyone has signed off. | Botany Access Supervision Tele Handler Operator |
| --- | --- | --- | --- | --- | --- |
| - 5 | - Access with vehicle onsite | Vehicles and plant moving around site | ###### 2 | - All persons entering site must be site inducted and follow all directions given in this induction including correct PPE - Follow all site rules / speed limits for particular site | Tele Handler Operator Site Supervision |
| - 6 | - Access to site by walking | Heavy plant and vehicle movements Slippery or uneven surface Hot, cold or wet conditions | 3 | - Follow directions as per site induction - Don’t work or walk behind vehicles or plant where you cannot be seen - Ensure eye contact made with plant operators prior to approaching plant work zone. - Ensure hi visibility clothing worn at all times onsite - Maintain high level of housekeeping in work area. - Wear appropriate clothing, sunscreen & PPE for weather and site conditions | Tele Handler Operator Site Supervision |

| Item | Job steps | Hazards | Risk Class/ Ranking | Controls | Name of persons responsible for work |
| --- | --- | --- | --- | --- | --- |
| - 7 | - Setup Tele Handler | Unstable ground & underground services | ###### 1 | - Ensure toolbox talk with customer’ supervisor is completed and JSRA is filled out prior to EWP setup. - Crane crew to obtain information from customer with regards to soil stability analysis to ensure load bearing pressure of soil able to hold EWP under outriggers and ensure sufficient packing used under outriggers. - Operator to obtain information from customer to ensure there are no underground services in setup area. | Tele Handler Operator Site Supervision |
| - 8 | - Isolate Tele Handler | Heavy plant and vehicle movements Pedestrian movements | 3 | - Ensure area around work zone is barricaded - Ensure no persons enter work zone without approval from operator. - Ensure all persons in area are wearing hardhat and hi visibility clothing | Tele Handler operator |
| - 9 | - Establish communication method | Lack of communication between operator & customer | 3 | - Operator and customer supervisor to discuss and agree on primary means of communication. Consideration to be taken to communication method used by other plant in area. - Where radios are being used ensure that a test is carried out prior to operation | Tele Handler Operator |

| Item | Job steps | Hazards | Risk Class/ Ranking | Controls | Name of persons responsible for work |
| --- | --- | --- | --- | --- | --- |
| - 10 | - Raise Basket to Tele Handler if fitted | Falling items, Falling persons | ###### 2 | - Ensure safe slinging practices are followed as identified in WorkCover Certification Course, Company Procedures, and site induction and AS 2550. - Ensure all persons in basket are wearing approved harness with lanyard attached to anchor point. - Ensure all items in basket are neat and secured in basket. | Tele handler Operator |
| - 11 | - Operating Tele handler | Tipping Tele Handler | 2 | - Refer to Operators manual and generic Genie GTH4514 Risk assessment. - Tele handler operator to check weight of load on forks and jib and load charts to ensure lift can be carried out within the SWL - Keep all movements steady and precise during all operations. - Ensure all persons slinging loads have the appropriate Dogman/Rigger WorkCover ticket. - Lower loads to ground level before travelling with loads. - Have loads secured before lifting for example Pallets strapped and tag lines on loads lifted with Jib. - Select correct attachment on Load management system or check attachment specific Load charts. | Tele Handler operator |

| - 1 | - Rigging and Lifting Beams with Tele handler | Securing beam to machine failing, | ###### 2 | - Using Rotator and Slippers ensue that all attachments are secure to machine and that all hoses intact with no leaks. - Ensure that no personal are underneath beam whilst being lifted into place unless authorised permission is given onsite. | Tele handler Operator |
| --- | --- | --- | --- | --- | --- |
| - 2 | - Barricading of Surroundings | Heavy plant and vehicle movements Pedestrian movements | ###### 3 | - Ensure area around work zone is barricaded using tape, barricades or witches hats. - Ensure no persons enter work zone without approval from operator. - Ensure all persons in area are wearing hardhat and hi visibility clothing and site specific PPE. | Tele handler Operator And Traffic Control |

Site Specific SWMS

| Item | Job steps | Hazards | Risk Class/ Ranking | Controls | Name of persons responsible for work |
| --- | --- | --- | --- | --- | --- |

| - 3 | - Traffic Control and management plan | Stopping vehicles and pedestrians whilst plant is in operation | ###### 2 | - Ensure to follow traffic management plan. - Ensure no traffic and persons do not enter site without approval from operator supervisor and Traffic controllers. - Please see traffic plan if attached with layout of how plan will be executed | Traffic Control And Tele handler Operator |
| --- | --- | --- | --- | --- | --- |
| - 4 | - Artificial lighting | Working during hours with out sun light | ###### 2 | - Must be in place for entire job whilst working in the night time on the area being worked on. - Day makers could be used as artificial light. | Traffic Control |

| Qualifications and experience required to complete the task | Personnel, Duties and Responsibilities (Supervisory staff and others) | Training Required to Complete Work |
| --- | --- | --- |
| National Certificate of Competency CN C1 or CO Trained on particular type of Tele Handler | Tele Handler Operator Operating Tele Handler, performing pre start logbook checks on Tele handler, involved in all toolbox meetings and filling out Risk Assessment forms, reporting all issues to site and Botany Access Management. | Site induction, Botany Access induction, inducted into this SWMS. |
| Training in OHS Risk Management for Supervisors and Managers. Experience in Crane and Rigging Industry. | Botany Access Supervision Supervision and regular safety checks carried out on EWP crews involved in operations onsite. Involved in preparation and improvements to Botany Access Management Systems. | Site induction, Botany Access induction. |
| Training in OHS Risk Management for Supervisors and Managers. OHS Consultation Course Experience Crane industry | Managing Director Ensure that company procedures are being followed as per OHS management system. Keep abreast of industry related regulations. Make changes to management systems where required. Review all site safety checks carried out onsite and put any corrective actions in place. Ultimately responsible in ensuring the safety and welfare of all employees whilst working onsite. | Site induction, Botany Access induction. |
| Engineering Details / Certificates / WorkCover Approvals: |  |  |
| Genie GTH4514 |  |  |

| This SWMS has been developed through consultation with our employees and has been read, understood and signed by all employees undertaking the works: |  |  |
| --- | --- | --- |
| Print Names: | Signatures: | Dates: |

PO Box 329, Botany NSW 1455

1356 Botany Rd Botany NSW 2019

Ph: 02 9666 3745

Fax: 02 9666 3564

E-mail: justin@botanyaccess.com.au

A.C.N.  073 330 239

READY TO START WORK SAFETY CHECKLIST

(TO BE COMPLETED PRIOR TO COMMENCEMENT OF MACHINE OPERATION)

Note: If you mark a NO box on the checklist you need to call Your Supervisor BEFORE commencing work.

Client:    CLARK & DOGGETT                      Name of machine operator: MICHAEL RIORDAN AND CHAD DOTTA

Site Location: PITT ST MALL

Site Contact:                                        Mobile:

Name of person who completed this checklist:       CHAD                                  Date:

Employer:  Botany Access                                         Signature:

DO YOU HAVE THE CORRECT PPE FOR YOUR TASK?

Safety boots Yes/No

Safety harness inspected and safe to use Yes/No

Hard Hat Yes/No

Eye protection and hearing protection Yes/No

High visibility vest or shirt Yes/No

Sunscreen Yes/No

Any other body protection required for this task Yes/No

HAVE YOU COMPLETED PROCEDURES AND FORMS?

Hazard identification and risk assessment Yes/No

Job safety analysis and SWMS Yes/No

Site evacuation procedures and location of first aid facilities Yes/No

Inspected EWP and completed logbook Yes/No

INDUCTION/COMPLIANCE

Have you completed general induction for construction occupational health and safety (greencard) Yes/No

Have you completed a site inspection Yes/No

Can you produce your workcover certificate of competency Yes/No

FIT FOR WORK

Do you agree you are not under the influence of alcohol or non prescribed drugs Yes/No

Do you agree you are not taking any prescribed drugs that may impair you ability to operate high risk plant/equipment Yes/No

SAFETY CONTROLS

Are specified hazard controls in place and working Yes/No

Have all necessary permits been obtained and/or sighted Yes/No

OPERATOR         Please return completed form to Botany Access office at conclusion of job

PO Box 329, Botany NSW 1455

1356 Botany Rd Botany NSW 2019

Ph: 02 9666 3745

Fax: 02 9666 3564

E-mail: justin@botanyaccess.com.au

A.C.N.  073 330 239

HAZARD IDENTIFICATION CHECKLIST- Utilise this to assist in completing your safe work method statement.

Client:       Site Location:

Date(s):                                         Name of machine operator:

| HAZARD | YES | NO | RISK ASSESSMENT 1,2 OR 3 | COMMENT/SAFETY CONTROLS |
| --- | --- | --- | --- | --- |
| Access & egress | X |  | 2 | BUNT OFF AREA PRIOR TO START |
| Asbestos |  | X |  |  |
| Biological |  | X |  |  |
| Confined space |  | X |  |  |
| Other trades working in area | X |  | 2 | BUNT OFF AREA PRIOR TO START |
| Dangerous materials |  | X |  |  |
| Demolition activity |  | X |  |  |
| Dust |  | X |  |  |
| Electrical incl. static |  | X |  |  |
| Environment: |  | X |  |  |
| Hot |  | X |  |  |
| Cold |  | X |  |  |
| Wet |  | X |  |  |
| Dark |  | X |  |  |
| Night |  | X |  |  |
| Excavations |  | X |  |  |
| Fire & explosion |  | X |  |  |
| Flooding & overflow |  | X |  |  |
| Gas, fumes, foul air |  | X |  |  |
| Hazardous equipment | X |  | 2 | OTHER EQUIPMENT IN AREA BUNT OFF AREA PRIOR TO START |
| Working over temporary buildings |  | X |  |  |
| Working radius of boom | X |  | 1 | WORK TO SPECIFICATIONS OF MACHINE SEE OPERATORS MANUAL |
| Hot surfaces |  | X |  |  |

OTHER COMMENTS

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

______________________________________________________________________________

OPERATOR        Please return completed form to Botany Access office at conclusion of job

PO Box 329, Botany NSW 1455

1356 Botany Rd Botany NSW 2019

Ph: 02 9666 3745

Fax: 02 9666 3564

E-mail: justin@botanyaccess.com.au

A.C.N.  073 330 239

| HAZARD | YES | NO | RISK ASSESSMENT 1,2 OR 3 | COMMENT/SAFETY CONTROLS |
| --- | --- | --- | --- | --- |
| Hot work |  | X |  |  |
| Hydraulic pressure | X |  | 2 | WEAR APPROPRIATE PPE CONTACT PIRTECH IMMEDIATLY |
| Manual handling | X |  | 3 | USE APPROPRIATE LIFTING METHOD |
| Moving machinery | X |  |  | BUNT OFF AREA PRIOR TO START |
| Sloping ground |  | X |  |  |
| Noise & vibration |  | X |  |  |
| Overhead services |  | X |  |  |
| Portable tools |  | X |  |  |
| Radiation (incl solar) |  | X |  |  |
| Trees |  | X |  |  |
| Railway lines |  | X |  |  |
| Suspended slab or floor |  | X |  |  |
| Pedestrians | X |  | 2 | BUNT OFF AREA PRIOR TO START |
| Traffic & vehicles | X |  | 2 | BUNT OFF AREA PRIOR TO START |
| Operating surface weight restrictions | X |  |  | WORK TO SPECIFICATIONS OF MACHINE SEE OPERATORS MANUAL |
| Unstable ground |  | X |  |  |
| Underground services |  | X |  |  |
| Waste disposal/mgmt |  | X |  |  |
| Work at height |  | X |  |  |
| Working downstream of water storage |  | X |  |  |
| Working over, near, on, in or under water |  | X |  |  |

OTHER COMMENTS

____________________________________________________________________________________________________________________________________________________________________________

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

______________________________________________________________________________

OPERATOR         Please return completed form to Botany Access office at conclusion of job

Signature